EXP 1 Simulation and Comparative Analysis of FDMA, TDMA and CDMA: Resource Allocation, User Capacity and Mobile-Generation Context

Name:hariharan                                                                                    Reg.no: 1912260124

Aim:

To simulate and compare the basic resource-allocation methods used in FDMA, TDMA and CDMA, and examine how their assumed user capacity and interference behaviour differ.

Objective 1:

To estimate and compare the illustrative number of users supported by FDMA, TDMA and CDMA using the specified system parameters.

MATLAB program:

>> % Comparison of FDMA, TDMA and CDMA

% Simplified illustrative model; not a full cellular-network simulation.

clear; clc; close all;

% Illustrative system parameters

totalBandwidthHz = 1e6;       % Total bandwidth: 1 MHz

fdmaChannelHz    = 200e3;     % FDMA channel width: 200 kHz

tdmaSlots        = 8;         % Number of TDMA slots per frame

processingGain   = 64;        % CDMA processing gain

targetSIRdB      = 7;         % Minimum acceptable CDMA SIR

% Capacity estimates under the stated assumptions

fdmaUsers = floor(totalBandwidthHz / fdmaChannelHz);

tdmaUsers = tdmaSlots;

targetSIR = 10^(targetSIRdB / 10);

cdmaUsers = floor(1 + processingGain / targetSIR);

fprintf('Illustrative maximum supported users\n');

fprintf('FDMA: %d users\n', fdmaUsers);

fprintf('TDMA: %d users\n', tdmaUsers);

fprintf('CDMA: %d users at a target SIR of %.1f dB\n\n', ...

cdmaUsers, targetSIRdB);

% CDMA SIR as the number of simultaneous users increases

K = 2:30;

sirLinear = processingGain ./ (K - 1);

sirDb = 10 * log10(sirLinear);

% Display a comparison table

method = ["FDMA"; "TDMA"; "CDMA"];

supportedUsers = [fdmaUsers; tdmaUsers; cdmaUsers];

results = table(method, supportedUsers);

disp(results);

% Plot estimated user capacity

figure;

bar(categorical(method), supportedUsers);

ylabel('Illustrative maximum supported simultaneous users');

title('FDMA, TDMA and CDMA: Capacity Estimate');

grid on;

% Plot estimated CDMA SIR as users are added

figure;

plot(K, sirDb, 'o-', 'LineWidth', 1.5);

hold on;

yline(targetSIRdB, '--r', 'Target SIR');

xlabel('Number of simultaneous CDMA users (K)');

ylabel('Estimated SIR (dB)');

title('Estimated CDMA SIR versus Number of Users');

grid on;

hold off;

Result:

Objective 2: CDMA SIR analysis:
To analyze how the estimated CDMA signal-to-interference ratio changes as the number of simultaneous users increases.

Result:

Objective 3: Mobile-generation comparison:
To relate representative multiple-access techniques to the evolution of mobile communication systems from 1G to 5G.

MATLAB Program:

generation = ["1G"; "2G"; "2G"; "3G"; "4G"; "5G"];

system = ["AMPS"; "GSM"; "IS-95"; "UMTS"; "LTE"; "5G NR"];

accessMethod = ["FDMA"; "TDMA"; "CDMA"; "WCDMA"; "OFDMA"; "OFDMA-based"];

keyIdea = ["Separate frequency channels"; ...

"Separate time slots"; ...

"Code-based user separation"; ...

"Wideband code-based access"; ...

"Orthogonal frequency resources"; ...

"Flexible OFDM-based radio access"];

evolutionTable = table(generation, system, accessMethod, keyIdea);

disp(evolutionTable);

Result:

Objective 4: Resource-allocation diagrams:
To visualize how FDMA, TDMA and CDMA assign frequency, time and code resources to multiple users.

MATLAB Program:

Time-frequency resource-allocation diagrams

figure;

tiledlayout(1,3);

% FDMA: users occupy different frequency channels

nexttile;

hold on;

colors = lines(4);

for u = 1:4

rectangle('Position',[0, u-1, 1, 0.8], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(0.5, u-0.6, sprintf('User %d',u), ...

'HorizontalAlignment','center');

end

xlim([0 1]); ylim([0 4]);

xlabel('Time');

ylabel('Frequency channel');

title('FDMA');

set(gca,'YTick',[]);

hold off;

% TDMA: users occupy different time slots on one frequency channel

nexttile;

hold on;

for u = 1:4

rectangle('Position',[u-1, 0, 0.8, 1], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(u-0.6, 0.5, sprintf('User %d',u), ...

'HorizontalAlignment','center');

end

xlim([0 4]); ylim([0 1]);

xlabel('Time slot');

ylabel('Frequency channel');

title('TDMA');

set(gca,'YTick',[]);

hold off;

% CDMA: users share the same time and frequency band using different codes

nexttile;

hold on;

for u = 1:4

rectangle('Position',[0, 0, 1, 1], ...

'FaceColor',colors(u,:), 'FaceAlpha',0.22, ...

'EdgeColor',colors(u,:), 'LineWidth',1.5);

end

text(0.5, 0.62, 'Users 1-4 share', 'HorizontalAlignment','center');

text(0.5, 0.40, 'band; different codes', 'HorizontalAlignment','center');

xlim([0 1]); ylim([0 1]);

xlabel('Time');

ylabel('Frequency band');

title('CDMA');

set(gca,'YTick',[]);

hold off;

sgtitle('Conceptual Time-Frequency Resource Allocation');

Result: