EXP3 MATLAB Analysis of Spectrum Bands, Propagation Loss and Cellular ++

Interference

Name: Hariharan                                                                                  Reg.no: 1912260124

# Aim:

# To compare a simplified free-space path-loss estimate across representative sub-1 GHz, mid-band and mmWave frequencies, and examine how distance and frequency reuse affect received signal and a simple interference measure.

# MATLAB Program — Path Loss, Received Power and Reuse Interference

# % Experiment 3: illustrative propagation and frequency reuse

# clear; clc; close all;

# % Representative bands and link-budget assumptions

# freqGHz = [0.7 3.5 28];

# freqMHz = freqGHz * 1000;

# bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"];

# txPowerdBm = 30;

# gtDbi = 0; grDbi = 0;

# distanceKm = logspace(log10(0.1),log10(2),200);

# distanceAtPointKm = 0.5;

# % Free-space path loss (d in km, f in MHz)

# fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

# lossAtPoint = fspl(freqMHz,distanceAtPointKm);

# rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

# linkTable = table(bandName,freqGHz(:),lossAtPoint(:),rxAtPoint(:), ...

# 'VariableNames',{'Band','Frequency_GHz','FSPL_dB','EstimatedRx_dBm'});

# disp(linkTable);

# % Path loss and received power over distance

# figure; tiledlayout(1,2);

# nexttile; hold on; grid on;

# for k=1:numel(freqMHz)

# plot(distanceKm,fspl(freqMHz(k),distanceKm),'LineWidth',1.5);

# end

# set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Path loss (dB)');

# legend(bandName,'Location','northwest'); title('Free-Space Path Loss');

# nexttile; hold on; grid on;

# for k=1:numel(freqMHz)

# rx = txPowerdBm + gtDbi + grDbi - fspl(freqMHz(k),distanceKm);

# plot(distanceKm,rx,'LineWidth',1.5);

# end

# set(gca,'XScale','log'); xlabel('Distance (km)'); ylabel('Estimated received power (dBm)');

# legend(bandName,'Location','southwest'); title('Illustrative Received Power');

# % Simple first-tier co-channel reuse estimate

# cellRadiusKm = 0.5;

# reuseN = [1 3 7 12];

# pathLossExponent = 3.5;

# reuseDistance = sqrt(3*reuseN) * cellRadiusKm;

# sirLinear = (reuseDistance/cellRadiusKm).^pathLossExponent / 6;

# sirDb = 10*log10(sirLinear);

# reuseTable = table(reuseN(:),reuseDistance(:),sirDb(:), ...

# 'VariableNames',{'ReuseClusterN','ApproxInterfererDistance_km','ApproxSIR_dB'});

# disp(reuseTable);

# figure; plot(reuseN,sirDb,'o-','LineWidth',1.5); grid on;

# xlabel('Reuse cluster size N'); ylabel('Approximate SIR (dB)');

# title('Simplified Co-Channel SIR versus Reuse Cluster Size');

# fprintf('At %.2f km, estimated received powers are:\n',distanceAtPointKm);

# disp(table(bandName,rxAtPoint(:),'VariableNames',{'Band','Rx_dBm'}));

# Result: