EXP5 MATLAB – Simulation and Throughput Analysis of LTE-Advanced Carrier Aggregation

Name:hariharan                                                                                  Reg.no: 1912260124

Aim

To simulate an LTE-Advanced wireless link using carrier aggregation and analyze the effect of aggregating multiple component carriers on available transmission bandwidth and estimated throughput using MATLAB.

Objective – 1: Model One Component Carrier

Define a single LTE-Advanced component carrier and estimate its throughput for a selected bandwidth and spectral efficiency.

MATLAB Program

clc; clear; close all;

BW1 = 20;                 % Component carrier bandwidth (MHz)
SE  = 4;                  % Spectral efficiency (bits/s/Hz)
Ncc = 1;                  % Number of component carriers

BWagg = BW1*Ncc;
Throughput = BWagg*SE;   % Mbps for MHz × bits/s/Hz

disp('Single Component Carrier')
disp(['Bandwidth = ',num2str(BWagg),' MHz'])
disp(['Estimated Throughput = ',num2str(Throughput),' Mbps'])

figure
bar(Ncc,Throughput)
grid on
xlabel('Number of Component Carriers')
ylabel('Estimated Throughput (Mbps)')
title('LTE-Advanced Single Carrier Throughput')

Result:

Objective – 2: Extend to Multiple Aggregated Component Carriers

Aggregate multiple component carriers and determine the resulting total bandwidth and estimated throughput.

MATLAB Program

clc; clear; close all;

BWcc = 20;                    % MHz per component carrier
SE = 4;                       % bits/s/Hz
CC = [1 2 3];                 % Number of component carriers

AggBW = BWcc .* CC;           % MHz
Throughput = AggBW .* SE;     % Mbps

disp('CC   Aggregated BW (MHz)   Estimated Throughput (Mbps)')
disp([CC' AggBW' Throughput'])

figure
bar(CC,Throughput)
grid on
xlabel('Number of Aggregated Component Carriers')
ylabel('Estimated Throughput (Mbps)')
title('LTE-Advanced Carrier Aggregation Throughput')

Result:

Objective – 3: Compare Estimated Throughput for Different Carrier Bandwidths

Procedure

MATLAB Program

clc; clear; close all;

BW = [5 10 15 20];            % Carrier bandwidths (MHz)
SE = 4;                       % bits/s/Hz
TP = BW .* SE;                % Estimated throughput (Mbps)

disp('Bandwidth (MHz)   Throughput (Mbps)')
disp([BW' TP'])

figure
plot(BW,TP,'o-','LineWidth',2)
grid on
xlabel('Carrier Bandwidth (MHz)')
ylabel('Estimated Throughput (Mbps)')
title('Throughput vs Component Carrier Bandwidth')

Result: